From  Ian Mackenzie

Had a little problem the other day that could have been catastrophic:

1. Symptoms. Display and desktop computer went intermittent, 'froze'  and sometimes had to be restarted from cold. 
Then two days ago the 'box' gave out a burning smell - only for a few minutes. 
But that got me worried!

2. Cause. Took the computer casing off - still no evidence of burning.
Took out power supply box and removed cover.
Ahhh - nasty burnt patch near fan and around main fuse.
Caused by mains voltage ignition of dust deposited near to power supply fan .
(I have attached photo of the nasty patch – see page 2)

3. Cure. Cleaned the board with meths and cotton buds.
Re-assembled.
I am buying an extra system fan to install in the casing(£10).
(Photo #2 on page 2 of two vents - power supply above and vacant vent below).
This should divert a large part of the dusty air sucked through the case.
I think a computer case really needs more than just its power supply fan.
(Could, of course, vacuum the study more often to remove source of contamination!).

4. Result. Computer now stable and 'sleeping' and 'hibernating' modes work properly.

Ian Mackenzie (aka the 'Hoover')

P.S. Checking the power supply only requires 'Meccano' skills with a screwdriver. 
(Keep the four sets of screws separate!)
Take care though not to touch the chips on the main motherboard - static on your hand could damage them.

_______________________________________

Images follow on page 2 ...

Burnt dust damage around the mains fuse(fan to left)


Main power supply fan vent above. Vacant system fan vent below.