- PLANNING WALKS

- Tracklogs:

- www.tracklogs.co.uk

- Disk-based software for designing walks – install on home computer.

- Ordnance Survey sheets bought disk by disk – Purbeck Explorer(£29.95)

- Can only work on desktop or laptop MS Vista/Windows7&8

- Shows the distance of your planned track bottom RH corner

- Can show cross section of track for ‘up and down’ distances

- Can add remarks or waypoint comments.

- Route can be exported to a dedicated GPS satnav  carrying the necessary map database e.g. Garmin.

- Can also produce ‘gpx’ for importing into Google Earth etc..

- There are other similar products e.g. MemoryMap and Anquet

- Google Earth

- Freely available satellite image of the Earth

- Can view any part of the world

- Can plot footpath trails on it – though, in my opinion, a bit of a tortuous process.

- Load files exported from smartphones/iphones or satnavs (see gpx section below).

- ViewRanger

- www.viewranger.com

- Only works with mobile phones and tablets as GPS/SatNav (see below)

- Versions for iphone/tablet, android and symbian(Nokia – now unsupported) operating systems.

- www.my.viewranger.com site provides planning facility with the maps that you own or have downloaded to your device(phone/tablet etc). You can plan walks on this site(PC or tablet) and synchronise with your GPS enabled tablet/mobile device.

- You buy mapping data by the 10kmx10km square(11p per tile)

- You can select  tiles using VR Map Chooser and download via an e-mail link after payment.

- As an example the whole Purbeck Explorer sheet 15 costs about £25 – N.B you only have to buy the land not the sea bits!

- Many highly detailed maps world-wide available for purchase.

- Dorset Explorer 3

- ***YOU SIMPLY MUST DOWNLOAD AND USE THIS HIGHLY INFORMATIVE SITE ON YOUR BROWSER

- www.explorer.geowessex.com through browser

- This is a DCC mapping database that displays a very wide range of information.

- It does not link to any GPS capability – mapping only.

- Can show OS maps and detail down to 6” scale (first top RH box under ‘?’)

- Seamless movement between scales

- Footpaths and Bridleways are shown as part of OS display.

- It also shows problems reported on rights of way

- Also holds data on what are adopted public highways.

- Rights of Way can be confirmed via ‘Layer Control’ (2nd top RH box below ‘?’)>Countryside>Rights of Way

- You CAN use your display in MS Windows to provide a map for planning although it’s a bit slow as it is on line.

- It can create a pdf map(printable) which is copyright protected – private use only!

- DCC Rights of Way Problem Report site:

- http://mapping.dorsetforyou.com/rightsofway/reportproblem/

- Can report a new problem – pinpointed by large scale mapping.

- Can show history of previous reports

- Check progress of previous reports

- ONLY reporting of official rights of way is possible i.e where official footpaths exist. It CANNOT be used to report non-ROW problems.

- GPX FILE FORMAT

- http://en.wikipedia.org/wiki/GPS_Exchange_Format

- This is the standard import/export format for transferring GPS data as text files

- Lots of embedded protocol but basically a sequential record of Lat/Long/Time/Elevation extracted from the satellite beam info.

- Your portable satnav device calculates the data from at least 3 beams and stores it.

- DATA LOGGER

- Device that only logs waypoints from satellite GPS readings it receives

- Has no screen or built in map database – so battery charge can last a long time.

- Can log may tracks and their points – so called ‘crumb trails’.

- An example is the G-sat DG-200 – 190,000 waypoints, 20 hours of operation. NOT unfortunately any more available.

- Canmore Electronics do make models (GT-730FL and 750FL) but reviews indicate the software interface is a bit flakey. Available from Amazon.

- You can get loggers to attach to your pet – if you are curious as to what they get up to!